# Hello Embedded World!

This short demo program represents “Hello World!” in an embedded environment by blinking an LED.

You will need the following to complete this exercise

- Atmel Studio 7.0

- AVRDUDE

- Arduino UNO R3

- Breadboard

- LED

- 330ohm Resistor

- Wires

- USB Power Supply (non-computer USB slot) – Recommended

Begin by following the directions to setup Atmel Studio 7.0, AVRDUDE, and a New Project.

Now enter the following code into your ASM file:

The instructions for this demo will be explained as we cover them throughout the semester.

Build the Program

- Menu: Build <project name>

- Review Output from the Build

  - We want to see “Build succeeded”

  - Build errors will be displayed somewhere above if the build fails

Program the Arduino

- Connect your Ardunio into a USB port on your computer

- Program using Menu: Tools\Arduino Programmer

- Output window should indicate successful programming

- Disconnect your Arduino from your computer

Build LED Circuit

- Start with a breadboard

- Place an LED with long leg (anode) in one of the power rails (red +) and short leg (cathode) in a row of the breadboard (mine is in row #10)

- Place one end of 330ohm resistor in ground rail (black -) and other leg in row with LED (row #10 in mine)

- Connect one wire (red in mine) to the positive power rail, and one wire (black in mine) to the negative ground rail of the breadboard

| LED | 330-ohm Resistor |
| --- | --- |

Connect Arduino to the Circuit

- Make sure power is not applied to the Arduino (i.e. disconnect the Arduino from the computer)

- Place the wire coming from the positive rail of the breadboard (red wire in my circuit) into the Port-D Pin-0 (labeled 0->RX)

- Place the wire coming from the negative rail of the breadboard (black wire in my circuit) into a ground of the Arduino (labeled GND)

Apply Power to the Arduino

- Apply power to your Arduino – LED should blink on/off each second

If possible, connect your Arduino to a non-computer USB power supply, i.e. mine is connected to a charging port on my USB hub. This helps prove to yourself that you programmed your MCU successfully, i.e. the program is running from your Arduino, not your computer.